Drone Energy & Routing Simulation — README

Overview

This project simulates the behaviour, energy usage, and routing decisions of a small electric delivery drone operating in a simplified 10×10 urban grid. It combines:

- Physics-based energy models (ODE-driven take-off/landing, analytical/simulated cruise power)

- Routing via Dijkstra’s algorithm

- Battery-aware trip planning with per-trip recharging

- Custom numerical tools such as linear interpolation and incremental root-finding

- Extensive plotting utilities for visualising energy usage, paths, and performance metrics

The simulation demonstrates how payload mass, path geometry, and flight energetics constrain a drone’s ability to complete multiple delivery trips.

Project Structure

| File | Description |
| --- | --- |
| main.py | Entry point; runs grid setup, ODE tables, root-finding demos, routing, and all major plots. |
| ode.py | Contains physical constants, energy/flight models, and ODE definitions for take-off, landing, and forward flight. Also includes cruise-energy solver and mass-scaled energy helpers. |
| root_finding.py | Custom incremental search + bisection solver; linear regression; exponential fitting; used for various numerical tasks. |
| interpolation.py | Simple piecewise linear 1D interpolator used for smoothing ODE outputs and trajectory plotting. |
| plotting.py | All visualisation utilities: take-off profile, payload vs. delivered mass, range/endurance, grid paths, and continuous interpolation. |
| read me.py | Original short explanation of project purpose (superseded by this README). |

How to Run the Simulation

1. Install Requirements

This project uses:

- typing

- random

- heapq

- numpy

- matplotlib

- scipy

2. Run the Simulation

Execute:

python main.py

You will see console output (trip summaries, root-finding results) and several pop-up plots.

3. Optional: Reproducible Runs

In main.py, set:

seed = (an integer value )

to make the randomly generated grid layout repeatable.

Simulation Workflow

This is the sequence executed in main.py:

1. Grid Generation

- Random warehouse location

- Random delivery cells with payloads (1–3 kg)

- Two random no-fly zones

- Printed ASCII grid map for reference

2. Energy Model Demonstrations

- Print table of takeoff/landing energy vs payload

- Compute and print crossover speeds (where drag = induced power)

- Plot takeoff altitude and energy profile (uses ODE + interpolation + root finding)

- Plot endurance and range vs payload (with exponential curve fits)

3. Routing & Trip Planning

The drone:

- Can carry up to 10 kg per trip

- Starts each trip with a fully recharged battery (≈0.0799 kWh)

- Uses Dijkstra-based shortest paths

- Ensures energy margin for return + landing before committing to a delivery

- Performs as many delivery trips as the battery and layout allow

Console output provides step-by-step trip logs.

4. Visualisations

Includes:

- Take-off ODE profile

- Payload vs total delivered mass

- Range/endurance vs payload

- Grid paths with nodes coloured by remaining battery

- Continuous interpolated trajectory for first trip

Key Algorithms & Models

Energy Models (in ode.py)

- Take-off & landing ODEs: thrust control, vertical drag, induced velocity, and electrical power.

- Cruise energy: integrated ODE with constant cruise tilt; fallback to analytical power model.

- Power model: induced + profile drag + fixed power.

- Energy scaling: takeoff/landing/cruise energy scaled linearly with mass.

Routing

- Full-grid Dijkstra per key point (warehouse + deliveries)

- Feasible-delivery selection based on energy margin

- Greedy nearest-delivery heuristic

- Per-trip battery logging

Custom Numerical Tools

- Incremental+binary-search root finder (no SciPy)

- Linear 1D interpolation

- Exponential regression using log-linear transform

Inputs & Outputs

Inputs (implicit/randomised)

- Grid size (10×10)

- Random warehouse and delivery locations

- Delivery payloads (1–3 kg)

- Random no-fly cells

- Drone parameters (mass, drag, battery, etc.) in ode.py

- Cruise speed and battery capacity in main.py

To customise, edit variables at the top of main.py.

Outputs

Console Output

- Grid layout

- Energy table (take-off/landing for sample payloads)

- Root-finding speed crossover results

- Detailed per-trip logs:

  - Moves, distances, loads, energy costs

  - Battery state after each step

  - Delivered payload

  - Full-trip summaries

Plots

- Take-off altitude & battery profile (ODE + interpolation)

- Payload vs total delivered mass per battery

- Range & endurance vs payload (plus exponential fits)

- Grid map with coloured drone paths

- Interpolated continuous flight trajectory